As a student at Union College, I am part of a community that values intellectual effort, curiosity and discovery.  I understand that in order to truly claim my educational and academic achievements, I am obligated to act with academic integrity. Therefore, I affirm that I carried out the work on this exam with full academic honesty, and I rely on my fellow students to do the same.

For this Exam, I understand that:

- I must work alone in writing out the solutions to the problems in this exam.

- I cannot copy solutions, in part or whole, to the problem on this exam from any person or resource.

- I cannot use any electronic resources to assist me in the solution to the questions on this exam except for my calculator to only performing appropriate calculations on the exam.

- I can use one page - single sided - of notes during the exam.  This one page of notes cannot contain any solutions to problems.  I must staple this page to the back of my exam at the end of the exam.

Signature:

Print Name:

Exam Date:

PROBLEM 1:  A horizontal  beam AB is supported by a pinned-end column CD, as shown in the figure.  The column CD is a solid steel bar (E=200 GPa) of square cross section having length 1.8m and side dimensions b=50mm ().

1a.  Using the freebody diagram below, determine the load in member CD.

1b.  Based upon the critical load of the column, determine the allowable load Q with respect to buckling.

PROBLEM 2:  Below is a ½-in pitch roller chain plate, as used on a bicycle chain.  Since a roller chain cannot transmit compression, the link is loaded in repeated axial tension (load fluctuates between 0 and a maximum force as the link goes from the slack side to the tight side of the chain) by pins that go through the two holes shown.

2a: Using the graph paper on the next page, draw the Modified Goodman diagram for this problem.  The link is made of carbon steel, heat-treated to give Su=140 ksi and Sy=110 ksi.  All surfaces are comparable to the “machined” category.  Use the conservative estimate of Notch Sensitivity q=1.0.

2b. Estimate the maximum tensile force to prevent yielding for static loading and the maximum tensile force for a static factor of safety of 2.

2c.  Estimate the maximum tensile force that will give infinite fatigue life.  Draw the fatigue stresses on the Modified Goodman Diagram in part 2a.  What is the tensile force for a fatigue factor of safety of 1.2?

Fatigue Surface Finish Factor Chart

Static Stress Concentration Factor